8085 ALP

| Store the data byte 32H into memory location 4000H MVI A, 32H STA 4000H HLT | Addition of two 8 bit numbers MVI A, 06 MVI B, 07 ADD B STA 8200 HLT |
| --- | --- |
| Addition of two 8 bit numbers MVI A, 06 MVI B, 07 ADD B STA 8200 HLT | Addition of two 8 bit numbers stored in memory LDA 8500 MOV B, A LDA 8501 ADD B STA 8502 HLT |
| Addition of two 8 bit numbers stored in memory and storing the carry LDA 8500 MOV B, A LDA 8501 ADD B STA 8502 MVI A, 00 ADC A STA 8503 HLT | Exchange the contents of memory locations 2000H and 4000H LDA 2000H MOV B, A LDA 4000H STA 2000H MOV A, B STA 4000H |
| Subtract the contents of memory location 4001H from the memory location 4000H and place the result in memory location 4002H. LXI H, 4000H MOV A, M INX H SUB M INX H MOV M, A HLT | Add the 16-bit number in memory locations 4000H and 4001H to the 16-bit number in memory locations 4002H and 4003H. The most significant eight bits of the two numbers to be added are in memory locations 4001H and4003H. Store the result in memory locations 4004H and 4005H with the most significant byte in memory location 4005H. LHLD 4000H XCHG LHLD 4002H MOV A, E ADD L MOV L, A MOV A, D ADC H MOV H, A SHLD 4004H HLT |
| Add the contents of memory locations 40001H and 4001H and place the result in the memory locations 4002Hand 4003H. LXI H, 4000H MOV A, M INX H ADD M INX H MOV M, A MVI A, 00 ADC A INX H MOV M, A HLT |  |

| Write a program to count number of l’s in the contents of D register and store the count in the B register. MVI B, 00H MVI C, 08H MOV A, D BACK: RAR JNC SKIP INR B SKIP: DCR C JNZ BACK HLT | Write a program to sort given 10 numbers from memory location 2200H in the ascending order. MVI B, 09 :"Initialize counter" START: LXI H, 2200H : Initialize memory pointer" MVI C, 09H :"Initialize counter 2" BACK: MOV A, M :"Get the number" INX H :"Increment memory pointer" CMP M :"Compare number with next number" JC SKIP :"If less, don’t interchange" JZ SKIP :"If equal, don’t interchange" MOV D, M MOV M, A DCX H MOV M, D INX H :"Interchange two numbers" SKIP: DCR C :"Decrement counter 2" JNZ BACK :"If not zero, repeat" DCR B :"Decrement counter 1" JNZ START HLT :"Terminate program execution" |
| --- | --- |
| Calculate the sum of series of even numbers from the list of numbers. The length of the list is in memory location 2200H and the series itself begins from memory location 2201H. Assume the sum to be 8 bit number so you can ignore carries and store the sum at memory location 2210H. Sample problem 2200H= 4H 2201H= 20H 2202H= l5H 2203H= l3H 2204H= 22H Result 22l0H= 20 + 22 = 42H LDA 2200H MOV C, A :"Initialize counter" MVI B, 00H :"sum = 0" LXI H, 2201H :"Initialize pointer" BACK: MOV A, M :"Get the number" ANI 0lH :"Mask Bit l to Bit7" JNZ SKIP :"Don’t add if number is ODD" MOV A, B :"Get the sum" ADD M :"SUM = SUM + data" MOV B, A :"Store result in B register" SKIP: INX H :"increment pointer" DCR C :"Decrement counter" JNZ BACK :"if counter 0 repeat" STA 2210H :"store sum" HLT :"Terminate program execution" | Search the given byte in the list of 50 numbers stored in the consecutive memory locations and store the address of memory location in the memory locations 2200H and 2201H. Assume byte is in the C register and starting address of the list is 2000H. If byte is not found store 00 at 2200H and 2201H. LX I H, 2000H :"Initialize memory pointer 52H" MVI B, 32H :"Initialize counter" BACK: MOV A, M :"Get the number" CMP C :"Compare with the given byte" JZ LAST :"Go last if match occurs" INX H :"Increment memory pointer" DCR B :"Decrement counter" JNZ BACK :"If not zero, repeat" LXI H, 0000H SHLD 2200H JMP END :"Store 00 at 2200H and 2201H" LAST: SHLD 2200H :"Store memory address" END: HLT :"Stop" |

| Multiplication of two 8-bit numbers MVI D, 00H MVI A, 00H LXI H, 4100H MOV B, M INX H MOV C, M LOOP: ADD B JNC NEXT INC D NEXT: DCR C JNZ LOOP STA 4200H MOV A, D STA 4201H HLT | Division of two 8-bit numbers LXI H, 4100H MOV B, M MVI C, 00H INX X MOV A, M NEXT: CMP B JC LOOP SUB B INR C JMP NEXT LOOP: STA 4200H MOV A, C STA 4201H HLT |
| --- | --- |
| Find the largest number from an array LXI H, 4200H MOV B, M INX H MOV A, M DCR B LOOP: INX H CMP M JNC AHEAD MOV A, M AHEAD: DCR B JNZ LOOP STA 4300H HLT | Find the smallest number from an array |

8086 ALP

| Write an assembly language program in 8086 microprocessor to divide a 16 bit number by an 8 bit number. MOV SI, 500 MOV DI, 600 MOV BL, [SI] INC SI MOV AX, [SI] DIV BL MOV [DI], AX HLT | Write an ALP to find factorial of number for 8086. MOV AX, 05H MOV CX, AX Back: DEC CX MUL CX LOOP back MOV [D000], AX HLT |
| --- | --- |
| The 8 data bytes are stored from memory location E000H to E007H. Write 8086 ALP to transfer the block of data to new location B001H to B008H. MOV BL, 08H MOV CX, E000H MOV EX, B001H Loop: MOV DL, [CX] MOV [EX], DL DEC BL JNZ loop HLT | Write a program to display string “LDRP Gandhinagar” in 8086. .model small .stack 100h .data String1 db "LDRP Gandhinagar", $ .code Main proc MOV AX, @data MOV DS, AX MOV AH, 09H MOV DX, offset String1 INT 21H MOV AH, 4CH INT 21H Main endp End Main |
| Write a program to reverse the given string for 8086. .model small .stack 100h .data String1 db “Computer Department LDRP”, $ Length dw $-String1-1 .code Main proc MOV AX, @data MOV DS, AX MOV SI, offset String1 MOV CX, Length ADD SI, CX Back: MOV DL, [SI] MOV AH, 02H INT 21H DEC SI LOOP Back MOV AH, 4CH INT 21H Main endp End Main | Write a program to multiply 2 numbers (16-bit data) for 8086. .model small .stack 100h .data Multiplier dw 1234H Multiplicant dw 3456H Product dw ? .code MULT proc MOV AX, @data MOV DS, AX MOV AX, Multiplicant MUL Multiplier MOV Product, AX MOV Product+2, DX MOV AH, 4CH INT 21H MULT endp End MULT |

| Sum of series of 10 numbers and store result in memory location total. model small .stack 100h .data List db 12,34,56,78,98,01,13,78,18,36 Total dw ? .code Main proc MOV AX, @data MOV DS, AX MOV AX, 0000H MOV CX, 0AH ; counter MOV BL, 00H ; to count carry MOV SI, offset List Back: ADD AL, [SI] JC Label Back1: INC SI LOOP Back MOV Total, AX MOV Total+2, BL MOV AH, 4CH INT 21H Label: INC BL JMP Back1 Main endp End Main | Write a program to find Largest No. in a block of data. Length of block is 0A. Store the maximum in location result. model small .stack 100h .data List db 80, 81, 78, 65, 23, 45, 89, 90, 10, 99 Result db ? .code Main proc MOV AX, @data MOV DS, AX MOV SI, offset List MOV AL, 00H MOV CX, 0AH Back: CMP AL, [SI] JNC Ahead MOV AL, [SI] Ahead: INC SI LOOP Back MOV Result, AL MOV AH, 4CH INT 21H Main endp End Main |
| --- | --- |
| Find number of times letter ëeí exist in the string ëexerciseí, Store the count at memory ans. .model small .stack 100h .data String db ëexerciseí, $ Ans db ? Length db $-String .code Main proc MOV AX, @data MOV DS, AX MOV AL,00H MOV SI, offset String MOV CX, Length Back: MOV BH, [SI] CMP BH, ëeí JNZ Label INC AL Label: INC SI LOOP Back MOV Ans, AL MOV AH, 4CH INT 21H Main endp End Main | Write an ALP to generate square wave with period of 200µs and address of output device is 55H for 8086 microprocessor. START: MOV AX, 01H OUT 30H, AX MOV BX, Count Label: DEC BX JNZ Label MOV AX, 00H OUT 30H, AX MOV BX, Count Label1: DEC BX JNZ Label1 JMP START |